| NO. |  | HX-RS-SMTP02 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NAME |  | Surface mount type platinum resistance temperature sensor |  |  |  |
| FEATURES |  | Temperature measurement for the surface of an object or equipment. |  |  |  |
| DESCRIPTION |  | 1.Products comply with IEC751 and EN60751 related standards. 2.Products designed to meet the provisions of the temperature sensor. 3.Components:Pt100. 4.Resistance:100Ω,at 0℃. 5.Accuracy: Class A,±（0.15+0.002*¦T¦）℃. 6.R/R ratio(ohm/ohm/℃):0.003851. 7.Working current:≤1mA. 8.insulating resistance:≥100MΩ. 9. Insulation voltage:≤500VDC. 10. Temperature range:-50～200℃(Or customer specified). 11. Size: 8*7.6*2.2mm . 12. Sensor structure: Single type. 13. Lead: 2wire,L=13mm. 14. Material:Cu. 15.Pressure: 0.6MPa. 16. Installation: Paste. 17. Measurement object: For measuring a flat surface. 18. Seal: IP68.Mechanical suppression. |  |  |  |
| ACCURACY AND RESISTANCE FOR TEMPERATURE |  |  |  |  |  |
| Temperature(℃); -50; 0; 100; 200. Resistance(Ω); 80.31; 100; 138.51; 175.86. Accuracy(℃); ±0.25; ±0.15; ±0.35; ±0.55. |
| ELECTRICAL CONNECTIONS |  |  |  |  |  |
| Pt100 Pt100 2-wire |  |  |  |  |  |
| PHYSICAL SAMPLE |  |  |  |  |  |
| DRAWING |  |  |  |  |  |
| HX-RS-SMTP02 |  |  |  |  |  |
| Product selection |  |  |  |  |  |
| Model |  |  | Code | Parameter | Note |
| HX-RS-SMTP02 |  |  | P | Pt100 |  |
| Accuracy |  |  | A | Class A |  |
|  |  |  | B | Class B |  |
| Temperature range |  |  | 1 | -20～85℃ |  |
|  |  |  | 2 | -50～150℃ |  |
|  |  |  | 3 | -50～200℃ |  |
|  |  |  | 4 | Customer designation |  |
| Lead | Structure |  | 2 | 2-Wire |  |